# IXC Content Delivery Guide

## 1. Technical Overview

Our projection environment utilizes a Pixera media server workflow. The space consists of four projection surfaces (North > East > South > West) driven by 11 projectors via Datapath Fx4 controllers.

[Click here to download] a zip file including:

- Example movie files

- Premiere Pro project file

- NotchLC codecs

- Canvas Mapping Images

## 2. Canvas Resolutions (Wall by Wall)

Please render content based on the specific pixel dimensions below. While all walls share the same height, widths vary due to physical mapping. Note: Even if creating continuous content, please render as 4 separate video files (one per wall) to manage file handling.

|  | Resolution (W x H) | Aspect Ratio | Projectors |
| --- | --- | --- | --- |
| North Wall | 3908px x 1080px | ~3.62:1 | 3 |
| East Wall | 5151px x 1080px | ~4.77:1 | 3 |
| South Wall | 3905px x 1080px | ~3.62:1 | 2 |
| West Wall | 5155px x 1080px | ~4.77:1 | 3 |
| Combined Canvas | 18119px x 1080px | ~16.78:1 | 11 |

## 3. File & Codec Specifications (Strict)

To ensure smooth playback on our servers, encoding with NotchLC is mandatory. It’s free, provides the image quality of ProRes 4444 but decodes on the GPU.  It’s free to download, the links are below. Choose “NotchLC” or .mov container in Premiere, AE, or Media Encoder.

- Required Format

- Codec: NotchLC (Notch Lossless Codec)

    - Download [NotchLC Codec for Adobe Suite: Windows]

    - Download [NotchLC Codec for Adobe Suite: Mac]

- Container: .MOV

- Frame Rate: 60 FPS

- Color Space: Rec.709 / sRGB

- Alpha Channel: Only if transparency is specifically required (NotchLC supports Alpha).

## 4. Content Guidelines & Best Practices

- Blend Zones: The vertical grid lines intersect where projectors overlap. Try to avoid placing fine text or logos directly on these vertical seams.

- Black Levels: We use projection mapping. Absolute black (#000000) projects as "dark grey" light. Avoid hard white borders at the very edge of the frame.

- 360° Horizon: If your content is intended to spin/circle around the room:

- Ensure the Right edge of the West Wall matches seamlessly with the Left edge of the North Wall.

- Sequence: North -> East -> South -> West -> (Back to North).

## 5. Naming Convention

Please deliver content using the following file naming structure:

[ProjectName]_[WallName]_[Resolution]_[FPS]_NotchLC.mov

Examples:

- OceanEvent_NorthWall_3908x1080_60fps_NotchLC.mov

- OceanEvent_EastWall_5151x1080_60fps_NotchLC.mov

## Summary Checklist for Creators

- [ ] Plugin: Is the NotchLC Adobe Plugin installed?

- [ ] FPS: Is the timeline set to 60 FPS?

- [ ] Resolution: Does the width match the specific wall map exactly?

- [ ] Codec: Is the output codec set to NotchLC?

## #CanvasMappings